! Note for this DLL

This DLL is the one used in the publication (Yang, Y., Yu, J., Bashir, M., & Li, C. (2026). Development and applicability of a novel fully coupled framework for floating wind energy systems. Ocean Engineering, 354, 124900.)

In this version of OpenF2A, the platform DOFs were solved together with other DOFs to consider the influence of platform on the tower dynamics. The restoring forces and added mass effects are considered as platform loads. In the AugMat matrix, these values are included in the platform associated terms, including platform-tower coupled terms.

This is different from what has been implemented in previous version of OpenF2A. The advantage is that the influence of platform on tower dynamics will be examined properly to correct tower’s natural frequency. However, there is a disadvantage of this implementation that numerical stability of the simulation cannot be guaranteed, which is mainly attributed to the platform acceleration transferred from AQWA. The backward difference of velocity is used to represent platform acceleration, which is valid for most cases, but the inevitably numerical error diminished the numerical stability.

This DLL works perfectly for the OC4 DeepCwind model, but it becomes invalid for the OC3 Hywind model. The same implementation strategy between OpenFAST and WEC-Sim is valid for the Hywind spar model, just because WEC-Sim can directly transfer the acceleration, rather than using the velocity difference like AQWA. Therefore, we suspect that numerical error of platform acceleration is the main reason for the numerical instability of the hybrid simulation framework OpenF2A.

In order to ensure the generalization of the OpenF2A interface, we have to release the source code of OpenF2A without correcting the tower dynamics. The platform dynamics can be accurately examined using the current tool. We hope that this code can be improved to address the problem of numerical instability shortly by someone else, or us.

Best regards,

Yang, 9th-March-2026.